Butterfly valve for long-distance water transport

To round off its existing range of butterfly valves, the KSB Group has launched the new Aporis double-offset butterfly valve with elastomer sealing element. This product is used for applications where huge volumes of water need to be transported and shut off reliably. Applications include systems for water transport, water treatment or in large industrial cooling circuits.

The valve is available with the face-to-face lengths specified in the EN 558/ISO 5752, Series 14, standard and in nominal sizes DN 300 to DN 2000. The maximum permissible operating pressure is 16 bar. The standard fluid temperature should range between 0 and 85 °C. The valve discs and bodies are made from epoxy-coated EN-GJS-400-15.

Two stainless steel stems hold the valve disc in position. Torque transmission from the actuating stem to the valve disc is ensured by a keyed connection. Neither of the stems come into contact with the fluid as they are surrounded by sealed protective sleeves. This keeps them dry and, in conjunction with the stainless steel reinforced PTFE bearing bushes, ensures extended, trouble-free service.

The flanged body with raised faces features integrally cast support feet which facilitate valve handling and storage. Thanks to the standardised top flanges to ISO 5211, the valve combines with a wide range of manual, electric, pneumatic and hydraulic actuators. The type series is approved for drinking water applications (DVGW, WRAS and ACS).

Photo: The new Aporis butterfly valves are used for applications where huge volumes of water need to be transported and shut off reliably.