New pneumatic actuator for diaphragm valves used in sterile applications

End of 2017 will see KSB’s Luxembourg subsidiary SISTO Armaturen S.A. launch a new double-acting or single-acting pneumatic actuator. It is designed for diaphragm valves used for sterile applications in the beverage, food and pharmaceutical industries.

A special technical feature of the new MD30-MD115 actuator range is that the actuator housing and the valve bonnet are made from a single piece of material. Compared with predecessor models, this design change substantially reduces the actuator’s overall height so that considerably less space is required to accommodate block-bodied multi-port valves. The actuators are also 45 percent lighter than the two-piece design. In addition to the stainless steel variant, the two largest actuators MD168 and MD202 are also available in a weight-saving aluminium design.

The actuator can be fitted to the valve body without nuts as the threads are integrated in the body, significantly facilitating mounting. The orientation of the air connection can be adjusted in 90° steps.

Accessories such as travel stop, positioner or proximity switches for actual-position feedback can be retrofitted by the operator as and when required. The actuators can be replaced with manual actuation bonnets at any time.

Thanks to their special design with enclosed, metal spiral supported diaphragm, Sisto C valves offer a long service life and can be operated at pressures as high as 20 bar.

Photo: Compared with predecessor models, the new MD30-MD115 actuators feature a notably reduced overall height. (KSB Aktiengesellschaft, Frankenthal)